#include <SPI.h>

#include <nRF24L01.h>

#include <RF24.h>

#define CSN_GPIO    8

#define CE_GPIO     7

#define RIGHT_FORWARD     5  // Left side motor forward

#define RIGHT_BACKWARD    4  // Left side motor backward

#define LEFT_FORWARD      3  // Right side motor forward

#define LEFT_BACKWARD     2  // Right side motor backward

// Hardware configuration

RF24 radio(CE_GPIO, CSN_GPIO);                           // Set up nRF24L01 radio on SPI bus plus pins 7 & 8

const byte Address[6] = "00001";

unsigned char Received_Command = 0,Speed_index = 0,Run_Stop_Mode = 0;   //  Run_Stop_Mode -> 0 = Stop , 1 = Run;

unsigned char Rx_Array[2];

unsigned int Run_Stop_Counter = 0;

void setup() {

Serial.begin(115200);

pinMode(RIGHT_FORWARD,OUTPUT);   //left motors forward

pinMode(RIGHT_BACKWARD,OUTPUT);   //left motors reverse

pinMode(LEFT_FORWARD,OUTPUT);   //right motors forward

pinMode(LEFT_BACKWARD,OUTPUT);   //right motors reverse

radio.begin();

radio.openReadingPipe(0, Address);

radio.setPALevel(RF24_PA_MIN);

radio.startListening();

Serial.println("START");

}

void loop()

{

if (radio.available())    // If the NRF240L01 module received data

{

delay(1);

//radio.read(&Received_Command, 1);

radio.read(&Rx_Array, 2);

Received_Command = Rx_Array[0];

Speed_index = Rx_Array[1];

Serial.print(Received_Command);

Serial.print(" , ");

Serial.println(Speed_index);

}

/***************** Speed control Logic ********************/

if(Run_Stop_Mode==0)    // Stop

{

digitalWrite(RIGHT_FORWARD,LOW);

digitalWrite(RIGHT_BACKWARD,LOW);

digitalWrite(LEFT_FORWARD,LOW);

digitalWrite(LEFT_BACKWARD,LOW);

Run_Stop_Counter++;

if(Run_Stop_Counter>=((5-Speed_index)*100))   // Speed_index = 1 -> Min speed,   5 -> Max Speed.

{

Run_Stop_Counter = 0;

Run_Stop_Mode = 1;

}

}

else if(Run_Stop_Mode==1)    // Run

{

Run_Stop_Counter++;

if(Run_Stop_Counter>=((Speed_index)*100))   // Speed_index = 1 -> Min speed,   5 -> Max Speed.

{

Run_Stop_Counter = 0;

Run_Stop_Mode = 0;

}

if(Received_Command == 1)            //move forward(all motors rotate in forward direction)

{

digitalWrite(LEFT_FORWARD,HIGH);

digitalWrite(RIGHT_FORWARD,HIGH);

}

else if(Received_Command == 2)      //move reverse (all motors rotate in reverse direction)

{

digitalWrite(LEFT_BACKWARD,HIGH);

digitalWrite(RIGHT_BACKWARD,HIGH);

}

else if(Received_Command == 3)      //turn right (left side motors rotate in forward direction, right side motors rotates in backward direction)

{

digitalWrite(LEFT_FORWARD,HIGH);

digitalWrite(RIGHT_BACKWARD,HIGH);

}

else if(Received_Command == 4)      //turn left (right side motors rotate in forward direction, left side motors rotates in backward direction)

{

digitalWrite(RIGHT_FORWARD,HIGH);

digitalWrite(LEFT_BACKWARD,HIGH);

}

else if(Received_Command == 0)      //STOP (all motors stop)

{

digitalWrite(RIGHT_FORWARD,LOW);

digitalWrite(RIGHT_BACKWARD,LOW);

digitalWrite(LEFT_FORWARD,LOW);

digitalWrite(LEFT_BACKWARD,LOW);

}

else                                  //STOP (all motors stop) , If any other command is received.

{

digitalWrite(RIGHT_FORWARD,LOW);

digitalWrite(RIGHT_BACKWARD,LOW);

digitalWrite(LEFT_FORWARD,LOW);

digitalWrite(LEFT_BACKWARD,LOW);

}

}

//delay(10);

}